Module 2 Unit 3

OPTICAL FIBRES – NUMERICAL PROBLEMS

SET - 1

- The acceptance angle of an optical fibre is 25°. Calculate RI of cladding if RI of core is 1.52.

- Find critical angle and acceptance angle for an optical fibre having core RI 1.5 and cladding 1.48.

- A fibre has core RI 1.5. Find its cladding RI if it is immersed in water giving acceptance angle of 8°. RI of water is 1.33.

- For an optical fibre, fractional RI is 0.0025 and core RI is 1.45. Determine its NA and acceptance angle.

- A fibre has acceptance angle of 25° and internal critical angle of 70°. Determine its core and cladding RI.

- Calculate number of allowed modes for a SI optical fibre having numerical aperture 0.2 and core radius 25 m. It is operated at 1550 nm.

- Calculate number of allowed modes for a GRIN fibre with above specifications.

- The core and cladding RI for a SI optical fire are 1.46 and 1.42 respectively. Find the normalized frequency and number of allowed modes if it is operated at 1.3 m. The core radius is 0.05 mm.

- What is the limiting radius for an optical fibre to serve as single mode at 8500 Å with numerical aperture of 0.025?

- An optical fibre has core radius of 5 m Will it act as a single mode fibre at 850 nm? Its core and cladding RI are 1.4 and 1.399 respectively.

- A step index optical fibre supports 1325 modes at an operating wavelength of 1.3 µm. Determine its core radius if the numerical aperture is 0.3.

- Determine the core radius for a GRIN fibre with above specifications.

------------------------------------------------------------------------------------------------------------------------------------------